VECTOR Digital Twin

Latency Test 1: ROS 2 → Unity

Date: 19 October 2026   |   Status: first run (before frame-rate fix)

# 1. What was measured

How long a ROS 2 message takes to reach the Unity twin and be applied in a Unity frame. This is the part of the chain the twin owns. The car, the Raspberry Pi and Wi-Fi are NOT included.

latency_probe.py → ros_tcp_endpoint → Unity (LatencyEcho) latency_probe.py ← ros_tcp_endpoint ← Unity (LatencyEcho)

Everything runs on the laptop, so one clock measures both ends (no clock sync needed).

# 2. How the test works

- latency_probe.py (ROS 2 node) sends a message every 0.1 s (10 Hz) for 60 s, stamped with its send time, on /twin/latency/ping.

- LatencyEcho.cs (Unity, on the LatencyTest object) sends the same message back on /twin/latency/pong in LateUpdate, i.e. after the twin cars have moved in that frame.

- When it comes back, the script measures: round trip = time received − time sent.

One-way latency ≈ round trip ÷ 2

# 3. Results

| Metric | Value | What it means |
| --- | --- | --- |
| Samples | 601 | Number of messages timed. |
| Round trip, mean | 63.96 ms | Average time to go to Unity and back. |
| One-way, mean | 31.98 ms | Half of the round trip: estimated time for one direction. |
| One-way, median | 32.33 ms | Middle value: half the messages were faster, half slower. |
| One-way, min | 1.56 ms | Fastest message: arrived just before a Unity frame, so it barely waited. |
| One-way, p95 | 52.60 ms | 95 % of messages were faster than this. |
| One-way, max | 71.58 ms | Slowest message: arrived just after a frame and waited for the next one. |
| Jitter (std, one-way) | 17.60 ms | How much the delay varies from message to message. |
| Unity frame time | 99.70 ms (~10 FPS) | Unity updated only once every 100 ms. |
| Lost messages | 0 / 601 | No message was lost: the link is reliable. |

All values are exactly as printed by latency_probe.py (FINAL block).

# 4. What it means

The connection is fast (min 1.56 ms) and reliable (0 lost). Almost all of the delay comes from Unity updating only 10 times per second: each message waits in line for the next Unity frame (0 – 100 ms). This is why the delay ranges from about 2 ms to about 72 ms.

Effect on the twin: at about 50 cm/s, a few tens of ms of delay means the twin car is drawn a few cm behind the real car, which is small compared with the 35 cm lane. The 10 FPS also makes the twin's motion look jerky.

Note: the waiting happens only on the way into Unity, so “round trip ÷ 2” is an approximation of the one-way delay.

# 5. Next steps

- Find why Unity runs at 10 FPS (Unity window not focused, a frame-rate cap in a script, Editor Interaction Mode, or laptop not on the NVIDIA GPU / not plugged in).

- Re-run the same test after the fix and compare (target: frame time about 16 ms or less).

- Later: one end-to-end video test (real car vs. twin on screen).

Raw data: ~/latency_step1.csv  (columns: seq, round_trip_ms, one_way_ms, unity_frame_ms)